Hardware Integration Documentation

Module: Bluetooth Remote Car Control using ESP32 and STM32

## 1.1 Objective

To implement wireless Bluetooth control of a two-sided motor-driven car using an ESP32 as the Bluetooth command interface and the STM32-based Custom Board as the real-time motor controller. The ESP32 receives commands from a Bluetooth-connected control application, forwards the commands to the STM32 through UART, and the STM32 interprets the commands to control motor direction and speed.

## 1.2 System Architecture

The system is divided into two control layers. The ESP32 provides the wireless Bluetooth interface and command forwarding layer, while the STM32 executes the vehicle-control logic. This separation keeps wireless communication independent from the motor-control functions.

Bluetooth Controller / Mobile Application → ESP32

ESP32 → UART command → STM32

STM32 → PWM + motor direction signals → Motor Driver

Motor Driver → Left and Right Motors

## 1.3 Components Used

| Component | Specification |
| --- | --- |
| Primary Controller | STM32-based Custom Board running MicroPython |
| Wireless Controller | ESP32 running Arduino code |
| Wireless Interface | Bluetooth / BLE command interface on ESP32 |
| Motor Driver | Dual-channel motor driver with ENA/ENB PWM inputs and direction inputs |
| Left Motor | DC motor controlled through LEFT_IN1 and LEFT_IN2 |
| Right Motor | DC motor controlled through RIGHT_IN1 and RIGHT_IN2 |
| STM32 UART | UART1 at 115200 baud |
| STM32 PWM | ENA on A6, ENB on A7, 1000 Hz |

## 1.4 STM32 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Purpose |
| --- | --- | --- | --- |
| UART1 TX | PA9 | UART | Transmit commands/communication |
| UART1 RX | PA10 | UART | Receive commands from ESP32 |
| Left Motor Enable (ENA) | A6 | PWM Output | Left motor speed |
| Right Motor Enable (ENB) | A7 | PWM Output | Right motor speed |
| Left Motor IN1 | B0 | GPIO Output | Left motor direction |
| Left Motor IN2 | B1 | GPIO Output | Left motor direction |
| Right Motor IN1 | C0 | GPIO Output | Right motor direction |
| Right Motor IN2 | C1 | GPIO Output | Right motor direction |

## 1.5 UART Communication Configuration

### 1.5.1 STM32 UART

UART1 is initialized at 115200 baud.

The source identifies PA9 as TX and PA10 as RX.

The STM32 continuously checks whether data is available using uart.any().

One byte is read at a time and converted to an uppercase UTF-8 command.

### 1.5.2 ESP32-to-STM32 Command Path

The ESP32 receives a Bluetooth command from the remote-control application.

The ESP32 forwards the command through its serial/UART connection to the STM32.

The STM32 interprets the command as F, B, L, R, or S.

The corresponding motor-control function is executed.

## 1.6 Motor Control Configuration

### 1.6.1 PWM Speed Control

The STM32 uses PWM to control the motor-driver enable inputs.

| Parameter | Value | Purpose |
| --- | --- | --- |
| PWM Frequency | 1000 Hz | Motor speed control |
| Straight Speed | 60000 / 65535 ≈ 91.6% | Forward motion |
| Turn Speed | 64000 / 65535 ≈ 97.7% | Turning |
| Backward Speed | 50000 / 65535 ≈ 76.3% | Reverse motion |

### 1.6.2 Motor Direction Control

| Command | Left Motor | Right Motor | Result |
| --- | --- | --- | --- |
| F | IN1=1, IN2=0 | IN1=1, IN2=0 | Forward |
| B | IN1=0, IN2=1 | IN1=0, IN2=1 | Backward |
| L | IN1=0, IN2=1 | IN1=1, IN2=0 | Turn Left |
| R | IN1=1, IN2=0 | IN1=0, IN2=1 | Turn Right |
| S | IN1=0, IN2=0 | IN1=0, IN2=0 | Stop |

## 1.7 Functional Operation Description

1. The ESP32 establishes the Bluetooth control interface and receives movement commands from the remote-control application.

2. The ESP32 forwards the received movement command to the STM32 over UART.

3. The STM32 initializes UART1 at 115200 baud and configures the motor-driver PWM and direction GPIOs.

4. During startup, the STM32 executes stop_car() so that both motors begin in a disabled state.

5. The STM32 continuously checks for incoming UART data.

6. When F is received, both motors are driven forward at STRAIGHT_SPEED.

7. When B is received, both motors are driven backward at BACKWARD_SPEED.

8. When L is received, the left motor is driven backward and the right motor forward at TURN_SPEED.

9. When R is received, the left motor is driven forward and the right motor backward at TURN_SPEED.

10. When S is received, PWM is set to zero and all motor direction outputs are cleared.

11. Unknown commands are rejected and reported as warnings.

12. The STM32 waits 10 ms between UART polling cycles.

13. If the STM32 program is interrupted, stop_car() is executed as an emergency motor shutdown.

## 1.8 STM32 MicroPython Code

from machine import UART, Pin, PWM
import time


# ============================================================
# UART1 - ESP32 COMMUNICATION
# ============================================================

# STM32H563 MicroPython build uses the predefined UART1 pins.
#
# UART1:
# PA9  -> TX
# PA10 -> RX
#
# ESP32 TX must connect to STM32 PA10
# ESP32 RX must connect to STM32 PA9

uart = UART(1, 115200)

print()
print("========================================")
print("   STM32H563 CAR CONTROLLER")
print("========================================")
print("UART1 : 115200 baud")
print("TX    : PA9")
print("RX    : PA10")
print("========================================")


# ============================================================
# MOTOR DRIVER
# ============================================================

# ENA / ENB
# These are PWM pins.

ENA = PWM(Pin('A6'))
ENB = PWM(Pin('A7'))

# PWM frequency
ENA.freq(1000)
ENB.freq(1000)


# ============================================================
# MOTOR DIRECTION PINS
# ============================================================

# LEFT MOTOR
LEFT_IN1 = Pin('B0', Pin.OUT)
LEFT_IN2 = Pin('B1', Pin.OUT)

# RIGHT MOTOR
RIGHT_IN1 = Pin('C0', Pin.OUT)
RIGHT_IN2 = Pin('C1', Pin.OUT)


# ============================================================
# MOTOR SPEED
# ============================================================

# duty_u16 range:
#
# 0     = 0%
# 32768 = 50%
# 65535 = 100%
#
# ------------------------------------------------------------
# STRAIGHT SPEED
# ------------------------------------------------------------

STRAIGHT_SPEED = 60000
# Approximately 91.6%


# ------------------------------------------------------------
# TURNING SPEED
# ------------------------------------------------------------

TURN_SPEED = 64000
# Approximately 95.0%


# ------------------------------------------------------------
# BACKWARD SPEED
# ------------------------------------------------------------

BACKWARD_SPEED = 50000
# Approximately 76.3%


# ============================================================
# PWM SPEED FUNCTION
# ============================================================

def set_speed(speed):

    if speed < 0:
        speed = 0

    if speed > 65535:
        speed = 65535

    ENA.duty_u16(speed)
    ENB.duty_u16(speed)


# ============================================================
# STOP CAR
# ============================================================

def stop_car():

    # Stop PWM
    ENA.duty_u16(0)
    ENB.duty_u16(0)

    # Stop left motor
    LEFT_IN1.value(0)
    LEFT_IN2.value(0)

    # Stop right motor
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(0)

    print("ACTION -> STOP")


# ============================================================
# FORWARD
# ============================================================

def move_forward():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # STRAIGHT SPEED
    set_speed(STRAIGHT_SPEED)

    print(
        "ACTION -> FORWARD | SPEED =",
        STRAIGHT_SPEED
    )


# ============================================================
# BACKWARD
# ============================================================

def move_backward():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # BACKWARD SPEED
    set_speed(BACKWARD_SPEED)

    print(
        "ACTION -> BACKWARD | SPEED =",
        BACKWARD_SPEED
    )


# ============================================================
# TURN LEFT
# ============================================================

def turn_left():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> LEFT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# TURN RIGHT
# ============================================================

def turn_right():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> RIGHT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# SAFE STARTUP
# ============================================================

stop_car()

print("CAR READY")
print("Waiting for commands...")
print()
print("F = Forward")
print("B = Backward")
print("L = Left")
print("R = Right")
print("S = Stop")
print()
print("STRAIGHT SPEED :", STRAIGHT_SPEED)
print("TURN SPEED     :", TURN_SPEED)
print("BACKWARD SPEED :", BACKWARD_SPEED)
print()


# ============================================================
# MAIN CONTROL LOOP
# ============================================================

try:

    while True:

        # Check if ESP32 sent something
        if uart.any():

            data = uart.read(1)

            if data:

                try:
                    command = data.decode('utf-8').upper()

                except:
                    command = ''

                if command:

                    print(
                        "UART COMMAND ->",
                        repr(command)
                    )


                    # ----------------------------------------
                    # FORWARD
                    # ----------------------------------------

                    if command == 'F':

                        move_forward()


                    # ----------------------------------------
                    # BACKWARD
                    # ----------------------------------------

                    elif command == 'B':

                        move_backward()


                    # ----------------------------------------
                    # LEFT
                    # ----------------------------------------

                    elif command == 'L':

                        turn_left()


                    # ----------------------------------------
                    # RIGHT
                    # ----------------------------------------

                    elif command == 'R':

                        turn_right()


                    # ----------------------------------------
                    # STOP
                    # ----------------------------------------

                    elif command == 'S':

                        stop_car()


                    # ----------------------------------------
                    # UNKNOWN COMMAND
                    # ----------------------------------------

                    else:

                        print(
                            "WARNING -> Unknown command:",
                            repr(command)
                        )


        time.sleep_ms(10)


# ============================================================
# EMERGENCY STOP
# ============================================================

except KeyboardInterrupt:

    stop_car()

    print()
    print("========================================")
    print("CAR CONTROLLER STOPPED")
    print("MOTORS DISABLED")
    print("========================================")

## 1.9 ESP32 Arduino Code

#include <BluetoothSerial.h>
#include "esp_system.h"

BluetoothSerial SerialBT;

HardwareSerial STM(2);

#define RXD2 16
#define TXD2 17


void setup()
{
    Serial.begin(115200);

    delay(1000);

    Serial.println();
    Serial.println("===== ESP32 BOOT =====");
    Serial.print("Reset reason: ");
    Serial.println(esp_reset_reason());

    SerialBT.begin("SOURYA_MICRO_CAR");

    STM.begin(115200, SERIAL_8N1, RXD2, TXD2);

    Serial.println("Bluetooth Ready");
    Serial.println("SOURYA_MICRO_CAR");
}

void loop()
{
    if(SerialBT.available())
    {
        char cmd = SerialBT.read();

        STM.write(cmd);

        Serial.print("Sent : ");
        Serial.println(cmd);
    }
}

## 1.10 Command Mapping

| Command | STM32 Action | Vehicle Behavior |
| --- | --- | --- |
| F | move_forward() | Car moves forward |
| B | move_backward() | Car moves backward |
| L | turn_left() | Car turns left |
| R | turn_right() | Car turns right |
| S | stop_car() | Car stops |

## 1.11 Speed Control and PWM

The STM32 converts the requested movement mode into PWM duty values applied to ENA and ENB. The set_speed() function clamps the requested duty value between 0 and 65535 before applying the same PWM value to both motor-enable channels.

ENA and ENB operate at 1000 Hz.

STRAIGHT_SPEED = 60000.

TURN_SPEED = 64000.

BACKWARD_SPEED = 50000.

A duty value of 0 disables the PWM output, while 65535 represents 100% duty according to the source comments.

## 1.12 Serial Output / Verification

STM32 startup: STM32H563 CAR CONTROLLER

STM32: UART1 : 115200 baud

STM32: CAR READY

STM32: Waiting for commands...

STM32: UART COMMAND -> 'F'

STM32: ACTION -> FORWARD | SPEED = 60000

STM32: ACTION -> STOP

STM32 emergency shutdown: CAR CONTROLLER STOPPED / MOTORS DISABLED

## 1.13 Known Limitation

The supplied implementation uses fixed speed values for forward, turning, and backward motion.

There is no proportional steering or independent left/right speed adjustment.

The STM32 polls UART rather than using interrupt- or DMA-based command reception.

Only the single-character commands F, B, L, R, and S are defined by the STM32 control logic.

The supplied source does not document the exact motor-driver model, motor electrical ratings, or battery specifications.

The Bluetooth application protocol implemented by the ESP32 is dependent on the supplied ESP32 source; the complete wireless-side behavior should be verified against that code.

## 1.14 Safety and Reliability Considerations

The STM32 performs a safe startup by calling stop_car() before accepting movement commands.

An explicit S command disables PWM and clears all motor direction outputs.

KeyboardInterrupt triggers stop_car() as an emergency software shutdown.

Ensure the motor-driver logic levels are compatible with the STM32 GPIO outputs.

Do not power motors directly from STM32 GPIO pins; use the appropriate motor-driver power path.

Ensure the motor driver, motors, STM32, and ESP32 share the required signal ground reference.

Use an appropriate power supply and protection scheme for the motor load.

## 1.15 Readiness for Future Enhancement

Add variable speed commands instead of fixed movement speeds.

Add gradual acceleration and deceleration to reduce mechanical stress.

Implement UART interrupts or DMA for more deterministic command reception.

Add command acknowledgements from STM32 to ESP32.

Add a communication watchdog that automatically stops the car if Bluetooth/UART commands are lost.

Add obstacle detection using the Custom Board's ultrasonic or IR sensors.

Integrate autonomous/manual hybrid driving modes.

Add telemetry from STM32 to ESP32 for battery, distance, and system status.

End of Document