Hardware Integration Documentation

Module: DHT11 Temperature and Humidity Monitoring using MicroPython on STM32

## 1.1 Objective

To interface a DHT11 temperature and humidity sensor with the STM32-based Custom Board using MicroPython, periodically acquire temperature and relative-humidity measurements, convert the temperature from Celsius to Fahrenheit, and display the readings through serial console logs. The program also handles sensor communication errors and allows safe termination through KeyboardInterrupt.

## 1.2 Components Used

| Component | Specification |
| --- | --- |
| Microcontroller | STM32-based Custom Board with MicroPython support |
| Temperature/Humidity Sensor | DHT11 digital sensor |
| Data GPIO | STM32 GPIO A7 |
| Pull-up Configuration | Internal Pin.PULL_UP enabled |
| Software | MicroPython |
| Python Modules | time, dht, and machine.Pin |

## 1.3 Electrical Specifications

### 1.3.1 Sensor Interface Configuration

The DHT11 data line is configured on GPIO A7.

A7 is initialized as a GPIO input with an internal pull-up using Pin.PULL_UP.

The native MicroPython dht.DHT11 driver is used to interface with the sensor.

The supplied program waits 2 seconds between measurements because the source identifies the DHT11 as a slow sensor.

The supplied source does not specify the sensor supply voltage, current consumption, cable length, or external pull-up resistor value.

## 1.4 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Configuration |
| --- | --- | --- | --- |
| DHT11 Data | A7 | GPIO Input | Internal pull-up enabled |

## 1.5 Detailed Connection Description

### 1.5.1 DHT11 Sensor Connection

Purpose

Provides digital temperature and humidity measurements to the STM32-based Custom Board.

Configuration

DHT11 data signal → GPIO A7.

A7 is initialized using Pin('A7', Pin.IN, Pin.PULL_UP).

The native MicroPython DHT11 driver is initialized using dht.DHT11(dht_pin).

A new measurement is triggered using sensor.measure().

Data Extraction

Temperature is obtained using sensor.temperature().

Relative humidity is obtained using sensor.humidity().

Temperature is additionally converted from Celsius to Fahrenheit in software.

## 1.6 Peripheral Configuration Summary

| Peripheral / GPIO | Configuration | Purpose |
| --- | --- | --- |
| A7 | GPIO Input + Internal Pull-Up | DHT11 data interface |
| DHT11 Driver | dht.DHT11 | Sensor measurement and data extraction |

## 1.7 Functional Operation Description

1. The program imports the time module, MicroPython dht module, and Pin from machine.

2. GPIO A7 is configured as an input with an internal pull-up resistor.

3. A native MicroPython DHT11 driver instance is created using the configured data pin.

4. The system prints an initialization message before entering the measurement loop.

5. The program waits 2 seconds before each measurement cycle.

6. sensor.measure() triggers a new DHT11 measurement.

7. The measured temperature in Celsius and relative humidity are extracted from the sensor driver.

8. The Celsius temperature is converted to Fahrenheit using the software conversion formula.

9. The temperature and humidity values are printed as formatted serial logs.

10. If a sensor communication or transmission error occurs, an OSError is caught and an error message is printed.

11. If the user stops the program with KeyboardInterrupt, a monitoring-stopped message is printed and the loop terminates.

## Code

In the MicroPython main script:

import time
import dht
from machine import Pin

# --- STM32 Hardware Configuration ---
# Set up the data pin. Replace 'PA8' with your actual STM32 pin name.
# Using Pin.PULL_UP acts as an internal backup pull-up resistor.
dht_pin = Pin('A7', Pin.IN, Pin.PULL_UP)

# Initialize the native MicroPython DHT11 driver
sensor = dht.DHT11(dht_pin)

print("DHT11 Sensor Initialized. Reading data...")

# --- Main Measurement Loop ---
while True:
    try:
        # CRITICAL: The DHT11 is a slow sensor. It requires at least 
        # a 1 to 2-second pause between readings, or it will throw an error.
        time.sleep(2)
        
        # Trigger a new hardware sensor transmission measurement
        sensor.measure()
        
        # Extract the calculated readings from the buffer
        temp_c = sensor.temperature()
        humidity = sensor.humidity()
        
        # Optional: Convert Celsius to Fahrenheit
        temp_f = (temp_c * 9/5) + 32
        
        # Print results neatly to the console logs
        print("-" * 40)
        print(f"LOG: Temperature = {temp_c}°C ({temp_f:.1f}°F)")
        print(f"LOG: Humidity    = {humidity}%")
        
    except OSError as e:
        # Catch reading transmission failures (e.g., loose wires, timeouts)
        print("LOG ERROR: Failed to read from DHT11 sensor. Checking connection...")
        
    except KeyboardInterrupt:
        print("\nMonitoring stopped by user.")
        break

## 1.8 Serial Output / Verification

DHT11 Sensor Initialized. Reading data...

LOG: Temperature = <temperature>°C (<temperature>°F)

LOG: Humidity    = <humidity>%

LOG ERROR: Failed to read from DHT11 sensor. Checking connection...

Monitoring stopped by user.

## 1.9 Known Limitation

The DHT11 requires a relatively long interval between measurements; the supplied program uses a 2-second delay.

The supplied source does not specify the exact DHT11 electrical specifications or physical wiring beyond the data GPIO.

The program is blocking during the 2-second measurement interval and does not perform other application tasks during that delay.

The error handler reports an OSError but does not implement an automatic sensor reset or retry strategy.

Temperature Fahrenheit conversion is performed in software and does not represent an additional sensor measurement.

## 1.10 Safety and Reliability Considerations

Ensure the DHT11 data interface is electrically compatible with the STM32 GPIO.

Use a common ground between the DHT11 sensor and Custom Board.

Do not apply a voltage above the STM32 GPIO input limit to A7.

Maintain appropriate wiring and connection integrity because the source explicitly handles possible loose-wire or timeout-related reading failures.

Respect the required DHT11 measurement interval to avoid repeated communication failures.

## 1.11 Readiness for Future Enhancement

Add configurable measurement intervals.

Add moving-average or other filtering for displayed sensor values.

Add minimum/maximum temperature and humidity tracking.

Add threshold-based alerts using the Custom Board's LED or buzzer outputs.

Transmit environmental measurements to the ESP32 for wireless monitoring.

Store measurements for logging, visualization, or environmental trend analysis.

End of Document