Hardware Integration Documentation

Module: IR Object Detection using MicroPython on STM32

## 1.1 Objective

To detect the presence or absence of an object using a digital IR sensor connected to the STM32-based Custom Board and provide a visual indication through a board LED. The MicroPython program also generates timestamped serial log messages whenever the detected state changes.

## 1.2 Components Used

| Component | Specification |
| --- | --- |
| Microcontroller | STM32-based Custom Board with MicroPython support |
| IR Sensor | Digital IR sensor; signal read through GPIO A0 |
| Board LED | LED controlled through GPIO A7 |
| Software | MicroPython |
| Python Modules | machine.Pin and time |

## 1.3 Electrical Specifications

### 1.3.1 GPIO and Signal Logic

IR sensor signal is configured as a digital GPIO input.

The IR sensor value is read using the Pin.value() method.

The supplied program assumes sensor_value = 0 (LOW) indicates an object detected.

The supplied program assumes sensor_value = 1 (HIGH) indicates no object detected.

The board LED is configured as a GPIO output.

## 1.4 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Operation |
| --- | --- | --- | --- |
| IR Sensor Signal | A0 | GPIO Input | Digital object detection |
| Board LED | A7 | GPIO Output | Visual status indication |

## 1.5 Detailed Connection Description

### 1.5.1 IR Sensor Connection

Purpose

Detects whether an object is present in front of the IR sensor.

Configuration

IR sensor signal is connected to GPIO A0.

A0 is configured as Pin.IN.

The program reads the digital sensor state using IR_SENSOR.value().

Signal Logic

LOW (0) → Object Detected

HIGH (1) → Object Not Detected

### 1.5.2 Board LED Connection

Purpose

Provides a visual indication corresponding to the IR sensor state.

Configuration

Board LED control is connected to GPIO A7.

A7 is configured as Pin.OUT.

The program drives the LED using BOARD_LED.value().

## 1.6 Peripheral Configuration Summary

| Peripheral / GPIO | Configuration | Purpose |
| --- | --- | --- |
| A0 | GPIO Input | Read IR sensor state |
| A7 | GPIO Output | Control board LED |

## 1.7 Functional Operation Description

1. The program imports the time module and Pin from machine.

2. GPIO A0 is configured as the IR sensor input and GPIO A7 as the board LED output.

3. A last_state variable is initialized to None so that the first valid sensor state is logged.

4. The system prints an initialization message and starts monitoring the IR sensor.

5. The raw digital sensor value is read continuously.

6. When the sensor reads LOW, the program enters the Object Detected state and logs the event only when the state changes.

7. When the sensor reads HIGH, the program enters the Object Not Detected state and logs the event only when the state changes.

8. The state-change logging prevents repeated identical messages from flooding the serial console.

9. A 50 ms delay is applied between readings.

10. If the program is stopped using KeyboardInterrupt, the LED is set to 0 and a shutdown message is printed.

## Code

In the MicroPython main script:

import time
from machine import Pin

# --- STM32 Hardware Configuration ---
# Replace 'PA3' and 'PA4' with your actual STM32 pin names
IR_SENSOR = Pin('A0', Pin.IN)          # IR Sensor signal line
BOARD_LED = Pin('A7', Pin.OUT)         # LED control pin

# Tracking variable to remember the last state (prevents log flooding)
# None = System start, 0 = Not Detected, 1 = Detected
last_state = None  

print("System Initialized. Monitoring IR Sensor...")

# --- Main Logic Loop ---
try:
    while True:
        # Read the raw sensor value. 
        # Most IR sensors output 1 (HIGH) for empty space, and 0 (LOW) for an obstacle.
        sensor_value = IR_SENSOR.value()
        
        if sensor_value == 0:
            # --- OBJECT DETECTED STATE ---
            BOARD_LED.value(1)  # Turn the LED OFF
            
            if last_state != 1:
                timestamp = time.ticks_ms() / 1000.0
                print(f"[{timestamp:.2f}s] LOG: Object Detected!")
                last_state = 1  # Update state
                
        else:
            # --- OBJECT NOT DETECTED STATE ---
            BOARD_LED.value(0)  # Keep the LED ON continuously
            
            if last_state != 0:
                timestamp = time.ticks_ms() / 1000.0
                print(f"[{timestamp:.2f}s] LOG: Object Not Detected")
                last_state = 0  # Update state

        # Tiny delay to prevent maxing out the STM32 core processing speed
        time.sleep_ms(50)

except KeyboardInterrupt:
    BOARD_LED.value(0)  # Safe shutdown
    print("\nProgram stopped by user.")

## 1.8 Serial Output / Verification

The program prints an initialization message followed by timestamped state-change messages. The supplied source generates messages in the following format:

System Initialized. Monitoring IR Sensor...

[time] LOG: Object Detected!

[time] LOG: Object Not Detected

Program stopped by user.

## 1.9 Known Limitation

The sensor logic is based on the behavior assumed in the supplied source: LOW indicates detection and HIGH indicates no detection.

The source comments contain a generic placeholder note about replacing pin names, but the actual code uses A0 and A7.

The program uses a 50 ms polling interval rather than interrupt-based detection.

The supplied code does not specify the exact IR sensor model, sensing distance, supply voltage, or current consumption.

## 1.10 Safety and Reliability Considerations

Ensure the IR sensor output is electrically compatible with the STM32 GPIO input.

Use a common ground between the Custom Board and the sensor.

Do not apply a voltage above the STM32 GPIO input limit to A0.

The 50 ms polling delay reduces unnecessary continuous processing.

The KeyboardInterrupt handler places the LED into a defined state during software shutdown.

## 1.11 Readiness for Future Enhancement

Add configurable detection thresholds if an analog IR sensor is used.

Add software debouncing or filtering for noisy digital sensor signals.

Replace polling with GPIO interrupt-based detection where supported.

Add buzzer or multi-LED alerts for object detection.

Transmit detection events to the ESP32 through UART.

Add event counters and detection-duration measurement.

End of Document