Hardware Integration Documentation

Module: Ultrasonic Distance Measurement using MicroPython on STM32

## 1.1 Objective

To measure the distance between the STM32-based Custom Board and an object using an ultrasonic sensor interface. The MicroPython program generates a 10-microsecond trigger pulse, measures the returning echo pulse duration, calculates the distance in centimeters using the speed of sound, and reports the result through the serial console.

## 1.2 Components Used

| Component | Specification |
| --- | --- |
| Microcontroller | STM32-based Custom Board with MicroPython support |
| Ultrasonic Sensor | Ultrasonic distance sensor with TRIG and ECHO digital interfaces |
| Trigger Interface | STM32 GPIO A0 |
| Echo Interface | STM32 GPIO A7 |
| Software | MicroPython |
| Python Interfaces | machine.Pin and machine.time_pulse_us |

## 1.3 Electrical Specifications

### 1.3.1 Sensor Interface and Timing

The trigger signal is generated on GPIO A0 using a digital output.

The echo signal is measured on GPIO A7 using a digital input.

The trigger pulse is held HIGH for 10 microseconds.

The echo measurement uses time_pulse_us(ECHO, 1, 30000), giving a 30 ms timeout.

The supplied source comments associate the 30 ms timeout with approximately 5 meters maximum measurement range.

The distance calculation uses 0.0343 cm per microsecond as the speed of sound and divides the measured round-trip distance by two.

## 1.4 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Operation |
| --- | --- | --- | --- |
| Ultrasonic Trigger | A0 | GPIO Output | 10 µs trigger pulse |
| Ultrasonic Echo | A7 | GPIO Input | Measure return-pulse duration |

## 1.5 Detailed Connection Description

### 1.5.1 Trigger Connection

Purpose

Initiates an ultrasonic measurement by transmitting a short digital trigger pulse.

Configuration

GPIO A0 is configured as Pin.OUT.

The trigger output is initialized LOW.

A 10-microsecond HIGH pulse is generated for each measurement.

The trigger is returned LOW immediately after the pulse.

### 1.5.2 Echo Connection

Purpose

Receives the sensor's return pulse so the time of flight can be measured.

Configuration

GPIO A7 is configured as Pin.IN.

time_pulse_us() measures the duration for which the ECHO signal remains HIGH.

A 30,000-microsecond timeout is applied.

A negative return value is treated as an out-of-range measurement or read error.

## 1.6 Peripheral Configuration Summary

| GPIO / Interface | Configuration | Purpose |
| --- | --- | --- |
| A0 | GPIO Output | Generate ultrasonic trigger pulse |
| A7 | GPIO Input | Measure ultrasonic echo pulse |
| time_pulse_us | 30 ms timeout | Measure echo duration in microseconds |

## 1.7 Functional Operation Description

1. The program imports time, Pin, and time_pulse_us from the MicroPython machine module.

2. GPIO A0 is configured as the trigger output and GPIO A7 as the echo input.

3. The trigger output is initialized LOW and held LOW for 2 milliseconds before measurement begins.

4. The system prints an initialization message.

5. The get_distance() function sets the trigger HIGH for exactly 10 microseconds and then returns it LOW.

6. The program measures the duration of the HIGH echo pulse on A7 with a 30 ms timeout.

7. If the pulse measurement returns a negative value, the function returns None to indicate an out-of-range or failed reading.

8. For a valid measurement, distance is calculated using the measured echo duration, the speed of sound, and the round-trip correction factor of two.

9. The calculated distance is rounded to two decimal places.

10. The main loop prints the measured distance or an out-of-range/no-object message.

11. A 500 ms delay is applied before the next measurement to avoid overlapping echoes.

12. KeyboardInterrupt terminates the measurement loop and prints a shutdown message.

## Code

In the MicroPython main script:

import time
from machine import Pin, time_pulse_us

# --- STM32 Hardware Configuration ---
# Replace 'PA5' and 'PA6' with your actual STM32 pin names
TRIG = Pin('A0', Pin.OUT)  # Trigger pin (Outputs the pulse)
ECHO = Pin('A7', Pin.IN)   # Echo pin (Reads the return pulse)

# Ensure trigger starts low
TRIG.value(0)
time.sleep_ms(2)

print("Ultrasonic Sensor Initialized. Reading distance...")

def get_distance():
    """Sends a pulse and calculates the distance in centimeters."""
    # 1. Send a precise 10-microsecond high pulse to trigger the sensor
    TRIG.value(1)
    time.sleep_us(10)
    TRIG.value(0)
    
    # 2. Measure the duration of the incoming high pulse on the Echo pin
    # time_pulse_us returns the time in microseconds, or -1/-2 on timeout
    duration_us = time_pulse_us(ECHO, 1, 30000) # 30ms timeout (~5 meters max)
    
    if duration_us < 0:
        return None  # Out of range or read error
        
    # 3. Calculate distance in cm
    # Speed of sound is ~0.0343 cm per microsecond. 
    # Divide by 2 because the wave travels to the object and back.
    distance_cm = (duration_us * 0.0343) / 2
    return round(distance_cm, 2)

# --- Main Measurement Loop ---
try:
    while True:
        distance = get_distance()
        
        # Print logs based on the sensor reading
        if distance is not None:
            print(f"LOG: Distance = {distance} cm")
        else:
            print("LOG: Out of range / No object detected")
            
        # Wait 500ms before the next reading to avoid overlapping echoes
        time.sleep_ms(500)

except KeyboardInterrupt:
    print("\nMeasurement stopped by user.")

## 1.8 Distance Calculation

The supplied program calculates distance using the following relationship:

Distance (cm) = (Echo duration (µs) × 0.0343 cm/µs) / 2

The division by two is used because the measured ultrasonic pulse represents the round-trip travel time from the sensor to the object and back to the sensor.

## 1.9 Serial Output / Verification

Ultrasonic Sensor Initialized. Reading distance...

LOG: Distance = <distance> cm

LOG: Out of range / No object detected

Measurement stopped by user.

## 1.10 Known Limitation

The supplied program treats any negative time_pulse_us() result as an out-of-range or read-error condition.

The source associates the 30 ms timeout with approximately 5 meters maximum range; actual sensor performance may vary.

The measurement loop is blocking and waits 500 ms between measurements.

The calculation uses a fixed speed-of-sound value of 0.0343 cm/µs and does not compensate for environmental conditions.

The supplied source does not specify the exact ultrasonic sensor model or its electrical voltage requirements.

## 1.11 Safety and Reliability Considerations

Ensure the ultrasonic sensor's ECHO output is electrically compatible with the STM32 GPIO input.

Do not apply an incompatible voltage level directly to GPIO A7.

Use a common ground between the sensor and Custom Board.

Keep the trigger and echo wiring secure to prevent intermittent measurements.

Avoid placing the sensor too close to surfaces or objects where the ultrasonic return may be unreliable.

## 1.12 Readiness for Future Enhancement

Add configurable measurement intervals and timeout values.

Add median or moving-average filtering for more stable distance measurements.

Add temperature compensation to improve speed-of-sound accuracy.

Add minimum/maximum distance thresholds for object detection.

Integrate distance readings with LEDs, buzzer, motors, or rover navigation logic.

Transmit distance measurements to the ESP32 for wireless monitoring or higher-level control.

End of Document