Hardware Integration Documentation

Module: Wi-Fi and Blynk IoT Remote Car Control using ESP32 and STM32

## 1.1 Objective

To implement Wi-Fi-based remote control of a motor-driven car using Blynk IoT and an ESP32 as the wireless command interface, while using the STM32-based Custom Board as the dedicated motor controller. The ESP32 receives joystick values from Blynk, converts the joystick position into movement commands, and sends the resulting command to the STM32 through UART. The STM32 then controls the motor-driver PWM and direction signals.

## 1.2 System Architecture

The application uses a two-level control architecture. The ESP32 manages Wi-Fi connectivity, Blynk IoT communication, joystick processing, and UART command transmission. The STM32 receives the resulting single-character command and performs the real-time motor-control operation.

Blynk IoT Joystick → Wi-Fi → ESP32

ESP32 → UART command → STM32

STM32 → PWM + direction signals → Motor Driver

Motor Driver → Left and Right DC Motors

## 1.3 Components Used

| Component | Specification |
| --- | --- |
| Primary Controller | STM32-based Custom Board running MicroPython |
| Wireless / IoT Controller | ESP32 running Arduino code |
| IoT Platform | Blynk IoT |
| Connection | Wi-Fi |
| User Control | Blynk Joystick widget |
| Joystick X Datastream | Virtual Pin V0 |
| Joystick Y Datastream | Virtual Pin V1 |
| STM32 UART | UART1 at 115200 baud |
| Motor Control | Dual-channel PWM + direction control |

## 1.4 Blynk Control Configuration

### 1.4.1 Joystick Datastreams

| Datastream | Virtual Pin | Data Type | Range | Default |
| --- | --- | --- | --- | --- |
| Joystick_X | V0 | Integer | 0–1023 | 255 |
| Joystick_Y | V1 | Integer | 0–1023 | 0 |

## 1.4.2 Joystick Processing

ESP32 stores X and Y values in joyX and joyY.

CENTER is set to 512.

DEADZONE is set to 150.

Y-axis movement has priority over X-axis movement in the command decision logic.

Y above CENTER + DEADZONE → F (Forward).

Y below CENTER - DEADZONE → B (Backward).

If Y is inside the deadzone, X below CENTER - DEADZONE → L (Left).

If Y is inside the deadzone, X above CENTER + DEADZONE → R (Right).

If both axes are within the deadzone → S (Stop).

A command is transmitted only when it differs from the previous command.

## 1.5 ESP32 Pin Assignment Summary

| Function | ESP32 GPIO | Peripheral | Purpose |
| --- | --- | --- | --- |
| STM32 UART RX | GPIO 16 | UART2 RX | Receive STM32 response/serial data |
| STM32 UART TX | GPIO 17 | UART2 TX | Transmit movement commands to STM32 |

## 1.6 STM32 Pin Assignment Summary

| Function | STM32 GPIO | Mode | Purpose |
| --- | --- | --- | --- |
| UART1 TX | PA9 | UART | Communication with ESP32 |
| UART1 RX | PA10 | UART | Receive movement commands |
| Left Motor ENA | A6 | PWM Output | Left motor speed |
| Right Motor ENB | A7 | PWM Output | Right motor speed |
| Left Motor IN1 | B0 | GPIO Output | Left motor direction |
| Left Motor IN2 | B1 | GPIO Output | Left motor direction |
| Right Motor IN1 | C0 | GPIO Output | Right motor direction |
| Right Motor IN2 | C1 | GPIO Output | Right motor direction |

## 1.7 UART Communication

STM32 UART1 operates at 115200 baud.

STM32 source identifies PA9 as TX and PA10 as RX.

ESP32 uses HardwareSerial port 2 with RXD2 = GPIO 16 and TXD2 = GPIO 17.

The ESP32 transmits the movement command as a single character.

The STM32 reads one byte, decodes it, converts it to uppercase, and maps it to a motor-control function.

TX and RX must be crossed between the ESP32 and STM32, with a common ground.

## 1.8 Motor Control Configuration

### 1.8.1 PWM Speed Control

| Parameter | Value | Approx. Duty |
| --- | --- | --- |
| PWM Frequency | 1000 Hz | — |
| STRAIGHT_SPEED | 60000 | 91.6% |
| TURN_SPEED | 64000 | 97.7% |
| BACKWARD_SPEED | 50000 | 76.3% |

## 1.8.2 Command Mapping

| Command | STM32 Function | Car Action |
| --- | --- | --- |
| F | move_forward() | Forward |
| B | move_backward() | Backward |
| L | turn_left() | Left turn |
| R | turn_right() | Right turn |
| S | stop_car() | Stop |

## 1.9 Functional Operation Description

1. The user moves the joystick in the Blynk dashboard or mobile application.

2. Blynk sends the X-axis value through V0 and the Y-axis value through V1.

3. The ESP32 receives each datastream update through BLYNK_WRITE(V0) or BLYNK_WRITE(V1).

4. The ESP32 calls processJoystick() to determine the required vehicle command.

5. The joystick values are compared against the center value of 512 and a deadzone of 150.

6. The ESP32 selects F, B, L, R, or S according to the joystick position.

7. sendCommand() prevents transmission of the same command repeatedly by comparing it with lastCommand.

8. The selected command is transmitted to the STM32 through ESP32 UART2.

9. The STM32 reads the command through UART1 and executes the corresponding motor-control function.

10. The STM32 controls motor speed through ENA/ENB PWM and direction through the motor-driver input pins.

11. The ESP32 periodically reports Wi-Fi status, RSSI, and Blynk connection status every 5 seconds.

12. The STM32 provides a safe startup by stopping the car before accepting commands and performs an emergency stop when interrupted.

## 1.10 STM32 MicroPython Code

from machine import UART, Pin, PWM
import time


# ============================================================
# UART1 - ESP32 COMMUNICATION
# ============================================================

# STM32H563 MicroPython build uses the predefined UART1 pins.
#
# UART1:
# PA9  -> TX
# PA10 -> RX
#
# ESP32 TX must connect to STM32 PA10
# ESP32 RX must connect to STM32 PA9

uart = UART(1, 115200)

print()
print("========================================")
print("   STM32H563 CAR CONTROLLER")
print("========================================")
print("UART1 : 115200 baud")
print("TX    : PA9")
print("RX    : PA10")
print("========================================")


# ============================================================
# MOTOR DRIVER
# ============================================================

# ENA / ENB
# These are PWM pins.

ENA = PWM(Pin('A6'))
ENB = PWM(Pin('A7'))

# PWM frequency
ENA.freq(1000)
ENB.freq(1000)


# ============================================================
# MOTOR DIRECTION PINS
# ============================================================

# LEFT MOTOR
LEFT_IN1 = Pin('B0', Pin.OUT)
LEFT_IN2 = Pin('B1', Pin.OUT)

# RIGHT MOTOR
RIGHT_IN1 = Pin('C0', Pin.OUT)
RIGHT_IN2 = Pin('C1', Pin.OUT)


# ============================================================
# MOTOR SPEED
# ============================================================

# duty_u16 range:
#
# 0     = 0%
# 32768 = 50%
# 65535 = 100%
#
# ------------------------------------------------------------
# STRAIGHT SPEED
# ------------------------------------------------------------

STRAIGHT_SPEED = 60000
# Approximately 91.6%


# ------------------------------------------------------------
# TURNING SPEED
# ------------------------------------------------------------

TURN_SPEED = 64000
# Approximately 95.0%


# ------------------------------------------------------------
# BACKWARD SPEED
# ------------------------------------------------------------

BACKWARD_SPEED = 50000
# Approximately 76.3%


# ============================================================
# PWM SPEED FUNCTION
# ============================================================

def set_speed(speed):

    if speed < 0:
        speed = 0

    if speed > 65535:
        speed = 65535

    ENA.duty_u16(speed)
    ENB.duty_u16(speed)


# ============================================================
# STOP CAR
# ============================================================

def stop_car():

    # Stop PWM
    ENA.duty_u16(0)
    ENB.duty_u16(0)

    # Stop left motor
    LEFT_IN1.value(0)
    LEFT_IN2.value(0)

    # Stop right motor
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(0)

    print("ACTION -> STOP")


# ============================================================
# FORWARD
# ============================================================

def move_forward():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # STRAIGHT SPEED
    set_speed(STRAIGHT_SPEED)

    print(
        "ACTION -> FORWARD | SPEED =",
        STRAIGHT_SPEED
    )


# ============================================================
# BACKWARD
# ============================================================

def move_backward():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # BACKWARD SPEED
    set_speed(BACKWARD_SPEED)

    print(
        "ACTION -> BACKWARD | SPEED =",
        BACKWARD_SPEED
    )


# ============================================================
# TURN LEFT
# ============================================================

def turn_left():

    # Left motor backward
    LEFT_IN1.value(0)
    LEFT_IN2.value(1)

    # Right motor forward
    RIGHT_IN1.value(1)
    RIGHT_IN2.value(0)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> LEFT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# TURN RIGHT
# ============================================================

def turn_right():

    # Left motor forward
    LEFT_IN1.value(1)
    LEFT_IN2.value(0)

    # Right motor backward
    RIGHT_IN1.value(0)
    RIGHT_IN2.value(1)

    # TURN SPEED
    set_speed(TURN_SPEED)

    print(
        "ACTION -> RIGHT | SPEED =",
        TURN_SPEED
    )


# ============================================================
# SAFE STARTUP
# ============================================================

stop_car()

print("CAR READY")
print("Waiting for commands...")
print()
print("F = Forward")
print("B = Backward")
print("L = Left")
print("R = Right")
print("S = Stop")
print()
print("STRAIGHT SPEED :", STRAIGHT_SPEED)
print("TURN SPEED     :", TURN_SPEED)
print("BACKWARD SPEED :", BACKWARD_SPEED)
print()


# ============================================================
# MAIN CONTROL LOOP
# ============================================================

try:

    while True:

        # Check if ESP32 sent something
        if uart.any():

            data = uart.read(1)

            if data:

                try:
                    command = data.decode('utf-8').upper()

                except:
                    command = ''

                if command:

                    print(
                        "UART COMMAND ->",
                        repr(command)
                    )


                    # ----------------------------------------
                    # FORWARD
                    # ----------------------------------------

                    if command == 'F':

                        move_forward()


                    # ----------------------------------------
                    # BACKWARD
                    # ----------------------------------------

                    elif command == 'B':

                        move_backward()


                    # ----------------------------------------
                    # LEFT
                    # ----------------------------------------

                    elif command == 'L':

                        turn_left()


                    # ----------------------------------------
                    # RIGHT
                    # ----------------------------------------

                    elif command == 'R':

                        turn_right()


                    # ----------------------------------------
                    # STOP
                    # ----------------------------------------

                    elif command == 'S':

                        stop_car()


                    # ----------------------------------------
                    # UNKNOWN COMMAND
                    # ----------------------------------------

                    else:

                        print(
                            "WARNING -> Unknown command:",
                            repr(command)
                        )


        time.sleep_ms(10)


# ============================================================
# EMERGENCY STOP
# ============================================================

except KeyboardInterrupt:

    stop_car()

    print()
    print("========================================")
    print("CAR CONTROLLER STOPPED")
    print("MOTORS DISABLED")
    print("========================================")

## 1.11 ESP32 Arduino Code

The credentials below are intentionally replaced with placeholders for documentation and distribution. Before uploading the sketch, replace every placeholder with the user's actual Blynk and Wi-Fi credentials.

#define BLYNK_TEMPLATE_ID "YOUR_BLYNK_TEMPLATE_ID"
#define BLYNK_TEMPLATE_NAME "YOUR_BLYNK_TEMPLATE_NAME"
#define BLYNK_AUTH_TOKEN "YOUR_BLYNK_AUTH_TOKEN"

#include <WiFi.h>
#include <BlynkSimpleEsp32.h>

char ssid[] = "YOUR_WIFI_SSID";
char pass[] = "YOUR_WIFI_PASSWORD";

#define RXD2 16
#define TXD2 17

HardwareSerial STM(2);

int joyX = 512;
int joyY = 512;

char lastCommand = 'S';

const int CENTER = 512;
const int DEADZONE = 150;

void sendCommand(char cmd)
{
    if(cmd != lastCommand)
    {
        STM.write(cmd);
        lastCommand = cmd;

        Serial.print("Sent : ");
        Serial.println(cmd);
    }
}

void processJoystick()
{
    Serial.print("X=");
    Serial.print(joyX);
    Serial.print("  Y=");
    Serial.println(joyY);

    char command = 'S';

    if(joyY > CENTER + DEADZONE)
    {
        command = 'F';
    }
    else if(joyY < CENTER - DEADZONE)
    {
        command = 'B';
    }
    else if(joyX < CENTER - DEADZONE)
    {
        command = 'L';
    }
    else if(joyX > CENTER + DEADZONE)
    {
        command = 'R';
    }
    else
    {
        command = 'S';
    }

    sendCommand(command);
}

BLYNK_WRITE(V0)
{
    joyX = param.asInt();
    processJoystick();
}

BLYNK_WRITE(V1)
{
    joyY = param.asInt();
    processJoystick();
}

void setup()
{
    Serial.begin(115200);

    STM.begin(115200, SERIAL_8N1, RXD2, TXD2);

    Serial.println();
    Serial.println("==============================");
    Serial.println("SOURYA REMOTE CAR");
    Serial.println("==============================");

    Serial.print("Connecting WiFi");

    WiFi.begin(ssid, pass);

    while(WiFi.status() != WL_CONNECTED)
    {
        delay(500);
        Serial.print(".");
    }

    Serial.println();
    Serial.println("WiFi Connected");
    Serial.print("IP : ");
    Serial.println(WiFi.localIP());

    Blynk.config(BLYNK_AUTH_TOKEN);

    Serial.print("Connecting Blynk");

    while(!Blynk.connect())
    {
        Serial.print(".");
        delay(500);
    }

    Serial.println();
    Serial.println("Blynk Connected");
    Serial.println("Ready");
}

void loop()
{
    Blynk.run();

    static unsigned long timer = 0;

    if(millis() - timer > 5000)
    {
        timer = millis();

        Serial.println("----------------------");

        if(WiFi.status() == WL_CONNECTED)
        {
            Serial.println("WiFi : Connected");
            Serial.print("RSSI : ");
            Serial.println(WiFi.RSSI());
        }
        else
        {
            Serial.println("WiFi : Disconnected");
        }

        if(Blynk.connected())
        {
            Serial.println("Blynk : Connected");
        }
        else
        {
            Serial.println("Blynk : Disconnected");
        }
    }
}

## 1.12 Blynk IoT Setup Procedure

Complete the Blynk configuration before uploading the ESP32 Arduino sketch.

### 1.12.1 Step 1: Create a New Template

Open a web browser and log in to the Blynk Console.

Click Templates in the left-hand navigation menu.

Click + New Template in the top-right corner.

Enter the basic settings: Name: Joystick Control (or any preferred name); Hardware: ESP32; Connection Type: Wi-Fi.

Click Done or Create to save the template.

### 1.12.2 Step 2: Create Datastreams for X and Y

Inside the newly created template, open the Datastreams tab.

Click + New Datastream and select Virtual Pin.

Create the X-axis datastream: Name = Joystick_X; Virtual Pin = V0; Data Type = Integer; Min Value = 0; Max Value = 1023; Default Value = 255.

Click Create.

Create the Y-axis datastream: Name = Joystick_Y; Virtual Pin = V1; Data Type = Integer; Min Value = 0; Max Value = 1023; Default Value = 0.

Click Create.

Click Save in the top-right corner of the template settings.

### 1.12.3 Step 3: Create a Device and Copy Credentials

Click Devices in the left menu and then click + New Device.

Select From template.

Choose the Joystick Control template and click Create.

Open the Device Info tab.

Locate the Firmware Configuration section.

Copy the Blynk Template ID, Template Name, and Auth Token for use in the Arduino sketch.

The credentials should be entered in this form:

#define BLYNK_TEMPLATE_ID "YOUR_BLYNK_TEMPLATE_ID"
#define BLYNK_TEMPLATE_NAME "YOUR_BLYNK_TEMPLATE_NAME"
#define BLYNK_AUTH_TOKEN "YOUR_BLYNK_AUTH_TOKEN"

### 1.12.4 Step 4: Configure the Joystick on the Dashboard

Return to the template and open the Web Dashboard, or use the Blynk mobile application.

Drag and drop the Joystick widget onto the dashboard.

Open the Joystick Settings.

Assign the X direction to Joystick_X (V0).

Assign the Y direction to Joystick_Y (V1).

Click Save to apply the settings.

Click Save and Ship in the upper-right section of the template screen to make the configuration live.

## 1.13 Credential Configuration Before Upload

IMPORTANT: The distributed .ino sketch must not contain the original Wi-Fi password, Blynk Auth Token, or private template credentials. The supplied documentation version uses placeholders. Replace them with the actual credentials only in the local working copy before compiling and uploading to the ESP32.

YOUR_BLYNK_TEMPLATE_ID → the Template ID generated by Blynk

YOUR_BLYNK_TEMPLATE_NAME → the template name created in Blynk

YOUR_BLYNK_AUTH_TOKEN → the device authentication token

YOUR_WIFI_SSID → the Wi-Fi network name

YOUR_WIFI_PASSWORD → the Wi-Fi network password

## 1.14 Serial Output / Verification

SOURYA REMOTE CAR

WiFi Connected

IP : <ESP32 IP address>

Blynk Connected

Ready

Sent : F / B / L / R / S

WiFi : Connected / Disconnected

RSSI : <signal strength>

Blynk : Connected / Disconnected

STM32: UART COMMAND -> 'F'

STM32: ACTION -> FORWARD | SPEED = 60000

STM32: ACTION -> STOP

## 1.15 Known Limitation

The joystick control uses fixed threshold and deadzone values.

The Y-axis command has priority over X-axis steering when both are outside their respective thresholds.

Only five single-character commands are defined by the STM32 controller.

The ESP32 does not transmit repeated identical commands.

The STM32 uses UART polling rather than interrupt- or DMA-based reception.

The supplied application does not implement a communication watchdog that automatically stops the vehicle if Wi-Fi, Blynk, or UART communication is lost.

The exact motor-driver model, motor ratings, battery characteristics, and physical wiring are not specified in the supplied source.

## 1.16 Safety and Reliability Considerations

The STM32 starts by calling stop_car(), keeping the motors disabled until a valid command is received.

The S command disables PWM and clears motor direction outputs.

KeyboardInterrupt on the STM32 triggers stop_car() as an emergency software shutdown.

A physical emergency-stop mechanism is recommended for a powered vehicle because software shutdown alone is not a complete safety mechanism.

Ensure ESP32-to-STM32 UART voltage levels are compatible and connect TX to RX with a common ground.

Do not power motors from MCU GPIO pins; use an appropriately rated motor driver and power supply.

Keep Wi-Fi and Blynk credentials private and do not publish the populated .ino sketch.

## 1.17 Readiness for Future Enhancement

Add a communication watchdog on the STM32 to stop the car if commands are not received within a defined timeout.

Add variable-speed joystick control instead of fixed speed values.

Add acceleration/deceleration ramps for smoother motion.

Add obstacle detection using the Custom Board ultrasonic or IR sensors.

Add battery-voltage telemetry to the Blynk dashboard.

Add live vehicle status, connection status, and sensor telemetry to Blynk.

Implement UART interrupts or DMA for deterministic command reception.

Add acknowledgement packets between STM32 and ESP32.

Add autonomous/manual mode selection through Blynk.

End of Document